- Comisiwn Brenhinol Henebion Cymru

- Royal Commission

- on the

- Ancient and Historical Monuments of Wales

Lanlash Farmhouse

County:  Carmarthenshire

Community: Llangathen

NGR: SN5707 2210

NPRN: 310170

Date of Survey: 18/11/2010,  8/12/2014

Surveyed by: Geoff Ward

Report Author: Geoff Ward

Illustrations: Geoff Ward

© Crown Copyright: RCAHMW 2010

Comments or queries regarding the content of the report should be made to the author:

RCAHMW, Plas Crug, Aberystwyth, Ceredigion SY23 1NJ

Tel: 01970 621214

World Wide Web: http//www.rcahmw.gov.uk

A house existed on this site from the early 15th century, according to the bard Lewis Glyn Cothi, who mentions it (Lan Lais) as belonging to Llwelyn ap Gwilym. His brother is mentioned in a poem adressed to Gwilym ap Thomas’s sons, the other being Henry ap Gwilym of (Lan Lais) and later Court Henry. A further poem to a first cousin Owain ap Thomas, of Gwilym ap Thomas’s sons, as shown on the family tree, specifically says that his grandfather Thomas Fychan was of (Lan Lais), which must extend the age of the house back to the beginnings of the 15th century and possibly earlier.

Lanlash retains elements of a medieval house, which was rearranged in the mid-17th when the ogee stop ceiling-beams to the cross-wing were inserted. The present house is a stone-built, 2-storey, ‘L’ shape plan house, with central entry to its front facade of 19th century appearance. It has in-line additions, which include an 18th century kitchen added at the upper end and a 19th century cow-house to the lower end. Part of the earlier house was converted for cattle with 18th century cattle lean-to at rear.

However evidence of an earlier building, can be seen in the thick walls, which are defined by stone foundation-arches at ground level. These arches first appear, according to Scoffield, about 1290 and can be seen in the 14th century Merchant Taylors’ Hall in London and also in the curtain wall of Rochester Castle. A nearer example is at Old Place, Llantwit-Major, Glamorgan. It seems that the damp position of this site made it necessary for the medieval builder to spread the load by using the foundation-arch technique in its wall construction. The fact that there are window loops to the ground-floor, and there is a first-floor entrance in the cross-wing indicates a fortified function. Its medieval plan has one metre thick walls consisting of a rectangular hall and a projecting solar cross-wing. The principal accommodation rooms were likely to be supported above a basement on timber beams rather than vaulted. A nearby example of a fortified first-floor hall/solar range structure is Court Henri, which dates from the 15th century, and was built by Henri ap Gwilym.

The medieval details at Llanlash are the first-floor semi-circular arched entry of red sandstone, in the cross-wing south-east side and a garderobe in its north-east gable-end. In the hall/basement there is a ground-floor entry in the north-east wall and an adjacent window loop with internal voussoir, but its gable-end and south-west wall are partly destroyed, and only the surviving foundation arches show its extent. There is voussoir evidence of former window loops at ground-floor, either end of the present cross-wing’s north-west wall. In the cross-wing’s south-west wall a square rebate indicates the position of an axial ceiling-beam at a lower level, which aligns with the first-floor entrance. It is probable that the cross-wing was originally heated by a fireplace over the present ground-floor one, which is an insertion of the 17th century, along with the present ceiling beams over the parlour rooms. The hall/basement’s gable-end wall has been partly destroyed and half of the present front wall has been rebuilt as a thinner wall with doorways for cattle. The doorway splay to the hall/basement is on the exterior, perhaps indicating it was once enclosed in a small structure linked with the first-floor doorway.

Description

The main south-west front facade is limewashed with end-chimneys and a slate roof. There is a central doorway with flanking sash windows to each floor under stone cambered voussoirs. Stone foundation arches are visible under the present ground-floor windows. A 3-door cow-house, situated in–line to the south-east has been formed from the former hall/basement and a further 3-door cow-house has been added in-line in the mid to late-19th century. The cow-house hall/basement doorways have stone voussoirs, while the cow-house has brick surrounds and arches. At the north-west there is an 18th century, 1 ½ storey, slate roofed, kitchen unit with end-chimney to inglenook, and a gable-end lean-to.

The rear north-east facade includes the cross-wing gable-end with batter at base, chimney stack, and a sash window on each floor with stone cambered arches, as in the front elevation. It also includes two ground-floor foundation arches. A blocked first-floor opening for a former garderobe is to the right with a stone segmental arch, partly damaged by alterations. This was an internal access into a former external timber-framed garderobe. To the left of the cross-wing is an 18th century cattle lean-to with ventilation slits. To the right is the 19th century service lean-to and rear of the kitchen unit. Originally the hall/basement doorway would have been visible against the left edge of the cross-wing.

Interior

The front doorway leads into a passage with living-room to left and front-parlour to right. The passage is taken out of the living-room, by a thin lathe & plaster partition to left. A mid-17th century partition to the right supports the living-room ceiling-beams. These ceiling-beams are cambered upwards and their medium chamfers have ‘ogee’ stops. The joists are square in section and with the ceiling-beams indicate a mid-17th century period. These beams continue in the rear-parlour only separated by a stud and plaster partition. The Living-room fireplace has been inserted with splayed sides, projecting both inwards and outwards from the end wall, and is slightly angled to the room. It has a chamferd timber lintel, which is sloped at its rear, but its left end may have been reduced slightly, to allow for a later doorway through to the kitchen. A cupboard to the right of the fireplace backs into a recess, which houses a range in the service-room, and was a former opening, which means the stud-partition has moved from its former line (see drawing). The front-parlour has a 19th century fireplace and plaster ceiling. It is divided by a narrow partition from a pantry under the stairs at the rear, formerly accessed by a doorway in the stud-partition (blocked). The 19th century stair behind the partition has chamfered and stopped capped newels, moulded hand-rail and stick balusters. Formerly the stair was in the entry passage. The outer wall of the stair contains the blocked hall/basement doorway. In this wall, where it is met by the abutting south-east cross wall of the front-parlour, part of the internal arch of a blocked loop-hole has been uncovered. Its full width is similar in size to the hall/basement doorway with similar voussoirs over.  The Rear-parlour room has a 19th century fireplace and gable-end window with plastered ceiling between ceiling-beams. An opening has been made into the service-room, where the cross-wing wall has a batter. This opening retains part of a blocked voussoir arch and splay for a loop hole. A doorway leads into the 18th century kitchen which has central and end ceiling-beams with narrow chamfers and run-out stops. Its joists have been replaced, but slight rebates show the earlier joists position, which are widely spaced.  The central beam has mortices for two studs and peg holes for wattle infill and may be re-used. There is a deep inglenook fireplace at the north-west gable-end with a chamfered timber lintel and bread oven (brick) to left side with a similar opening to the right side (now altered), perhaps for a small cooking oven. There is a stair to left of fireplace, which was formerly boxed in. The former basement/hall may once have extended from the line of the mid-17th century stud partition to the end of the wall of the former cow-house. Its rear wall retains two foundation arches and two doorways one interrupting a foundation arch.

At first-floor, the parlour-wing retains a similar mid-17th century stud and plaster partition, as on the ground-floor, dividing it into two rooms. There is a 19th century fireplace in the north-east gable-end and a blocked opening over the living-room fireplace, perhaps the site of an original fireplace? In the loft above, the gable walling around the chimney is infill, indicating there was no original gable and that the chimney stack would have been visible as a tall stack. Unfortunately the original roof has been replaced by late-19th century softwood tie-beam trusses, still with pegged joints. This indicates major works were being undertaken at this time and probably dates the present sawn soft-wood roof timbers. Indeed, most of the outbuilding roofs seem to have been replaced at the same time. There are large stone tiles in the garden, which would have been used prior to the mid-19th century. The gable of the former cowhouse retains one ventilation slit in its former hay loft.

Comment

This enigmatic structure shows that we have a lot to learn about the development and history of the medieval houses of this area between Carmarthen and Llandeilo. Information regarding early features which were formerly obscured by later alterations were revealed whilst minor repairs to the fabric were ongoing. These features changed the interpretation considerably and were recorded simultaneously by the owner and the Commission and have enhanced our interpretation.

References:  John A Lloyd, The Early History of Court Henry, Dryslwyn, Carmarthenshire Historian, Vol 16, 1979.  Lewis Jones,  Hist. Carms. Homes & their families, 1987, P28. Francis Green, Unpublished History of the Green Family of Court Henry, c1910, in possession of Thomas Lloyd.

Geoff Ward 20/01/2015.

Gwaith Lewis Glyn Cothi. The Poetical Works of Lewis Glyn Cothi.  Oxford. Cymmrodorian 1837, p 185,186

Gwaith Lewis Glyn Cothi. The Poetical Works of Lewis Glyn Cothi.  Oxford. Cymmrodorian 1837, p 185,186

Gwaith Lewis Glyn Cothi. The Poetical Works of Lewis Glyn Cothi.  Oxford. Cymmrodorian 1837, Family Tree

Gwaith Lewis Glyn Cothi. The Poetical Works of Lewis Glyn Cothi.  Oxford. Cymmrodorian 1837, p 211

Scoffield, J. The Buildings of London, 1984, p99.

Gwaith Lewis Glyn Cothi. The Poetical Works of Lewis Glyn Cothi.  Oxford. Cymmrodorian 1837, p 185,186

The owner Chris Plummer uncovered a relieving arch in the front south-west wall, which appears to cut through the present floor level, perhaps indicating a former window to an open hall. However this conflicts with the window loop on the opposing rear wall. There is also a blocked doorway with timber lintel opposite the hall/basement entrance, which could have formed a cross-passage.

? The South-Wales genealogist Francis Green recalls a “massive oak beam” being removed in about 1890-1900?.